LED-TURN OFF-ON

SIGNAL-BUTTON

MULTIPLE LED’S

PROGRAM -4

PYTHON - CAMERA

FLASK

Summary of the code:

This Python script uses the Flask web framework to create a web server that interacts with GPIO components (a button and three LEDs) on a Raspberry Pi. The user can access the server through a browser to check the button state or control the LEDs by sending specific HTTP requests.

Steps:

1) Import required libraries: `Flask` for web application creation, `Button` and `LED` from `gpiozero` for GPIO hardware control.

2) Initialize a `Button` object on GPIO pin 26 with a debounce time of 0.05 seconds to prevent false triggers.

3) Initialize three `LED` objects connected to GPIO pins 17, 27, and 22 and store them in a list.

4) Turn off all LEDs initially using a `for` loop.

5) Create a Flask web application instance.

6) Define a route at `/` which returns a welcome message when accessed.

7) Define a route `/push-button` that checks if the physical button is pressed and returns the result.

8) Define a route `/led/<int:led_number>/state/<int:state>` to turn ON or OFF a specific LED based on the values passed in the URL.

9) Start the Flask application, making it accessible from any IP on the network.

RASPBERRY PI –Project Setup

Summary of the code:

This Python script runs on a Raspberry Pi and automates the process of detecting motion using a PIR (Passive Infrared) sensor. When motion is detected and it lasts longer than a threshold time, the Raspberry Pi camera takes a picture. This picture is saved in a folder, logged into a file, and then sent via email to a predefined recipient. The system uses LEDs for visual feedback and ensures that photos are not taken too frequently.

Steps:

1) Import required libraries to handle GPIO (motion sensor, LED), camera control, email sending, time control, and file operations.

2) Define utility functions:

- `take_photo()` to capture an image using the camera.

- `update_photo_log_file()` to log photo file paths.

- `send_photo_by_email()` to email the captured image.

3) Initialize global variables for motion tracking, timing, paths, and thresholds.

4) Setup email credentials securely from a hidden password file.

5) Setup and configure the camera resolution and orientation.

6) Ensure the destination folder and log file are ready (create/delete as needed).

7) Initialize GPIO components: PIR motion sensor and LED.

8) Define callbacks:

- `motion_detected()` to record the start time and turn the LED on.

- `motion_finished()` to check motion duration, take a photo if valid, and send it via email.

9) Bind the callbacks to the PIR sensor events.

10) Run the program continuously with `pause()` to keep it active.

def motion_finished():

global last_time_photo_taken

led.off()

motion_duration = time.time() - time_motion_started

if motion_duration > MOVEMENT_DETECTED_THRESHOLD:

if time.time() - last_time_photo_taken > MIN_DURATION_BETWEEN_PHOTOS:

last_time_photo_taken = time.time()

print("Taking a photo and sending it by email")

photo_file_name = take_photo(camera, CAMERA_FOLDER_PATH)

update_photo_log_file(LOG_FILE_NAME, photo_file_name)

send_photo_by_email(yag, photo_file_name)

FINAL CODE

from flask import Flask

import os

# ---------------- Constants ----------------

CAMERA_FOLDER_PATH = "/home/pi/photos_final_project"

LOG_FILE_NAME = os.path.join(CAMERA_FOLDER_PATH, "photo_logs.txt")

previous_line_counter = 0

# ---------------- Flask App Initialization ----------------

app = Flask(__name__, static_url_path=CAMERA_FOLDER_PATH, static_folder=CAMERA_FOLDER_PATH)

# ---------------- Routes ----------------

@app.route("/")

def index():

return "Hello"

@app.route("/check-photos")

def check_photos():

global previous_line_counter

line_counter = 0

if os.path.exists(LOG_FILE_NAME):

last_photo_file_name = ""

with open(LOG_FILE_NAME, "r") as f:

for line in f:

line_counter += 1

last_photo_file_name = line.strip()

difference = line_counter - previous_line_counter

previous_line_counter = line_counter

message = f"{difference} new photos since you last checked <br/>"

message += f"Last photo: {last_photo_file_name} <br/>"

message += f"<img src=\"{last_photo_file_name}\">"

return message

else:

return "No photo available"

# ---------------- Run Server ----------------

app.run(host="0.0.0.0")